|  | 506/507/508 - HA389427 |  |
| --- | --- | --- |
|  | Chapter | 3 |
|  | Terminal Description |  |
|  | Contents Page |  |

- Power Terminals

- Auxiliary Terminals

- Switches

- Potentiometers

- LEDS

# Terminal Description

| ### Control Terminals |  |  |  |
| --- | --- | --- | --- |
| Terminal | Function | Description | Notes |
| 1 | Tacho Feedback | Motor Mounted Tachogenerator Input Terminal. | 200V dc Max. |
| 2 | 0V-Signal Common | Control Signal Reference. | Must NOT be Earthed (Grounded). |
| 3 | Run | Digital Input to Enable Controller. Connect to 0V to Run in Speed. Connect to + 10V to Run in Current. | Input Impedance -> 20k Ohm |
| 4 | Speed Setpoint | Analogue Input, 0v to +10V = 0 to 100% Speed. | Input Impedance - 66k Ohm |
| 5 | +10V Reference | Analogue Output, +10V Reference for Speed and Current Setpoints. | Load - 2mA |
| 6 | Current Limit | Analogue Input, 0 to +10V=0 to 100% Current | Input Impedance - 100k 0hm |

- This is a non-isolated product.  The control connections are not isolated from the AC supply.   Any connection to earth or use of earth (ground) referenced components either deliberately or unintentionally will cause permanent damage to the controller.

| ### Power Terminals The product may be connected phase - to - phase provided the resultant voltage does not exceed the maximum supply voltage specified. |  |  |  |
| --- | --- | --- | --- |
| Terminal | Function | Description | Notes |
| L1 (L) | AC Input Live | AC Supply Terminal Line 1 or Live |  |
| L2 (N) | AC Input Neutral | AC Supply Terminal Line 2 or Neutral |  |
| F+ | Field Positive | Field Output Positive |  |
| F- | Field Negative | Field Output Negative |  |
| A- | Armature Negative | Armature Output Negative |  |
| A+ | Armature Positive | Armature Output Positive |  |
| Grd | Protective Earth (Ground) | Drive Module Protective Earth (Ground) |  |
| Grd | Protective Earth (Ground) | Motor Protective Earth (Ground) |  |

| ### Auxiliary Terminals * The power must be supplied from an independent power supply which is not earth (ground) referenced. |  |  |  |
| --- | --- | --- | --- |
| Terminal | Function | Description | Notes |
| A1 | Auxiliary Supply | Auxiliary Relay Supply | +16V at 10mA |
| A2 | 0V - Signal Common | Signal Reference | Must not be earthed (grounded) |
| A3 | Zero Speed | Zero Speed Output | 24V at 50mA open Collector * |
| A4 | Health | Health Output | 24V at 50mA open Collector * |
| A5 | Zero Speed Level | Zero Speed Output Level Trim | For other levels see Table 3.1 |
| A6 | Auxiliary Setpoint | Auxiliary Direct Speed Setpoint | +10V @ 100K Full Speed Input |

Table 3.1

| External Resistor | Terminal Volts at Zero Speed | 180Va | 90Va |
| --- | --- | --- | --- |
| -- | 10.75V | 6% | 12% |
| 4k7 | 5.5V | 3% | 6% |
| 2k2 | 2.5V | 2% | 4% |
| 1k | 2V | 1% | 2% |

The zero speed detector operates from the Controller Output voltage, the default level is set to 6% of 180V dc.

| ### Switches |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Switch |  |  | Off |  | On |
| SW1 | Imax. |  | 506 - 0.25 to 1.5A | Imax. | 506 - 0.5 to 3.0A |
|  | Imax. |  | 507 - 0.5 to 3.0A | Imax. | 507 - 1.0 to 6.0A |
|  | Imax. |  | 508 - 1.0 to 6.0A | Imax. | 508 - 2.0 to 12.0A |
| SW2 |  | ½ Speed Demand |  | Full Speed Demand |  |
| SW3 |  | ¼ Speed Feedback |  | Full Speed Feedback |  |
| SW4 |  | Tacho Feedback |  | Armature Feedback |  |

| ### Potentiometers |  |  |
| --- | --- | --- |
| P1 | RAMP UP | Setpoint ramp adjustment clockwise to increase time. |
| P2 | RAMP DOWN | Setpoint ramp adjustment clockwise to increase time. |
| P3 | SPEED STABILITY | Speed loop stability clockwise increase response. |
| P4 | CURRENT CALIBRATION | Output current calibration clockwise to increase output. |
| P5 | IR COMPENSATION | Speed droop compensation armature voltage feedback. |
| P6 | MINIMUM SPEED | Clockwise to increase compensation. Minimum speed at zero setpoint. (Can also be used as zero speed trim anticlockwise to reduce speed at zero setpoint). |
| P7 | MAXIMUM SPEED | Maximum speed calibration clockwise to increase speed. |

| ### LEDS |  |
| --- | --- |
| POWER-ON | Indicates power supply is present on terminals. (Note:- This LED does not indicate that the CORRECT voltage is applied). |
| HEALTH | When illuminated the controller is unhealthy due to either an overcurrent trip or a stall trip condition. Should normally be "OFF". |