|  | 584SV Series Frequency Inverter - HA463660 |  |
| --- | --- | --- |
|  | Chapter | 2 |
|  | An Overview of the Inverter |  |
|  | Contents Page |  |

- Component Identification

- Control Features

- Understanding the Product Code

- Functional Overview

# An Overview of the Inverter

## Component Identification

Figure 2-1  General Features

## Control Features

The Inverter is fully-featured when controlled using the 5841 Operator Station (or a suitable PC programming tool).

The `General’ control features below are not available when the unit is controlled using the analogue and digital inputs and outputs.

| General | Output Frequency | Selectable 0-120Hz, 240Hz or 480Hz |
| --- | --- | --- |
|  | Switching Frequency | Constant Torque: 3khz for all units Quadratic Torque: 3khz for all units |
|  | Voltage Boost | - 0-25% (selectable Auto or Manual) |
|  | Flux Control | - 1. V/F control with linear or fan law profile 2. Sensorless vector with automatic flux control and slip compensation |
|  | Slip Compensation | 0-Limit Frequency/24 |
|  | Skip Frequencies | 4 skip frequencies with adjustable skip band width |
|  | Preset Speeds | 8 presets with programmable ramp rates |
|  | Stopping Modes | Ramp, ramp with hold, coast, dc injection, fast stop |
|  | Ramps | Symetric or asymetric ramp up and down rates |
|  | Raise/Lower | Programmable MOP function |
|  | Jog | Programmable jog speed |
|  | Logic Functions | 10 programmable 3 input logic function blocks performing NOT, AND, NAND, OR, NOR and XOR functions |
|  | Value Functions | 10 programmable 3 input value function blocks performing IF, ABS, SWITCH, RATIO, ADD, SUB, RATIO, TRACK/HOLD, and BINARY DECODE functions |
|  | Diagnostics | Full diagnostic and monitoring facilities |
| Protection | Trip Conditions | Output short line to line, and line to earth Overcurrent > 220% I x t overload 50-105% (adjustable) Heatsink overtemperature Motor Thermistor overtemperature Overvoltage and undervoltage |
|  | Current Limits | Adjustable 50%-150% 180% shock load limit |
|  | Voltage/ Frequency Profile | Constant torque Fan Law (base frequency adjustable) |
| Inputs/ Outputs | Analogue Inputs | 4 user-configurable: Speed setpoint/trim ±10V, 0-10V, 2-10V, 0-5V, 1-5V Speed setpoint 4-20, 20-4, 0-20 or 20-0mA Current loop 4-20, 20-4, 0-20 or 20-0mA Torque limit 0-10V |
|  | Analogue Outputs | 2 user-configurable: Speed 0-10V and 0-20mA Load ±10V |
|  | Digital Inputs | 8 x 24V dc inputs (user configurable) |
|  | Digital Outputs | 3 relay contacts (volt-free) |

Table 2-1  Control Features

## Understanding the Product Code

The 584SV Inverter is fully identified using a ten block alphanumeric code which records how the Inverter was calibrated, and its various settings when despatched from the factory.

The product code appears as the "Model No.”. Details of each block of the Product Code can be found in the Product Manual, however, the following blocks below are substituted for the Types 8, 9 and 10.

| Block No. | Variable | Description |
| --- | --- | --- |
| 2 | XXXX | Four numbers specifying the constant torque power rating kW. For quadratic torque see Chapter 11: “Technical Specification” - Electrical Ratings - Power Circuit (quadratic torque). Horsepower ratings below as specified within NEC/NFPA-70. 0900= 90kW @ 380V (150hp @ 460V) 1100=110kW @ 380V (150hp @ 460V) 1320=132kW @ 380V (200hp @ 460V) 1600=160kW @ 380V (250hp @ 460V) 1800=180kW @ 380V (300hp @ 460V) 2000=200kW @ 380V (300hp @ 460V) 2500=250kW @ 380V (400hp @ 460V) 2800=280kW @ 380V (450hp @ 460V) |
| 3 | XXX | Three numbers specifying the nominal input voltage rating: 400 380 to 460V (±10%) 50/60Hz |
| 4 | XXXX | Four digits specifying the mechanical package including livery and mechanical package style: First two digits Livery 00 Standard Eurotherm Drives livery 01-99 Defined customer liveries Third digit Mechanical packaging style 1 Standard (IP2X), protected panel mounting (terminals:: IP00) Fourth digit Operator Station 1 Standard product (must be 1) - 5841 Operator station fitted |
| 9 | XXX | Three digits specifying the cooling fan auxiliary supply: 115 115V fan option supplied. 230 230V fan option supplied. |

## Functional Overview

Figure 2-2  584SV High Power AC (Types 8, 9 and 10) simplified block diagram